Software Requirements Specification (SRS)

Telecommunications Network Analysis and Optimization for Local Communities in Cameroon

Document Version: 1.0
Date: April 2026
Prepared for: Academic Defense, Application Development with Python Course
Platform: Kali Linux

Document Revision History

| Version | Date | Author | Description | Validation |
| --- | --- | --- | --- | --- |
| 0.1 | April 2026 | Student | Initial draft | Pending |
| 0.2 | April 2026 | Student | Functional requirements added | Pending |
| 1.0 | April 2026 | Student | Complete SRS document | Pending |

1. Introduction

1.1 Purpose

This Software Requirements Specification (SRS) document provides a comprehensive description of the Telecommunications Network Analysis and Optimization System designed for local communities in Cameroon. The system aims to assess, analyze, and optimize wireless network performance using Kali Linux built-in tools, addressing the documented Quality of Service (QoS) challenges faced by Cameroonian telecom users .

1.2 Document Scope

This document covers:

- Functional and non-functional requirements for the network analysis system

- Hardware and software specifications for the Kali Linux implementation

- Data collection and analysis methodologies

- Optimization strategies for local community networks

- Regulatory compliance considerations for Cameroon

1.3 Intended Audience

| Audience | Purpose |
| --- | --- |
| Academic Reviewers | Evaluate project methodology and technical depth |
| Thesis Advisor | Assess project scope and feasibility |
| Local Community Stakeholders | Understand network improvement recommendations |
| Telecommunications Regulators (ART) | Review compliance with QoS monitoring standards |

1.4 Definitions and Acronyms

| Term | Definition |
| --- | --- |
| ART | Agence de Régulation des Télécommunications (Cameroon's Telecommunications Regulatory Board) |
| QoS | Quality of Service - Measures of network performance including throughput, latency, and reliability |
| RSSI | Received Signal Strength Indicator - Measurement of power present in received radio signal |
| BSSID | Basic Service Set Identifier - MAC address of wireless access point |
| ESSID | Extended Service Set Identifier - Network name |
| SRS | Software Requirements Specification (this document) |
| PATNUC | Projet d'Accélération de la Transformation Numérique du Cameroun |
| horst | Highly Optimized Radio Scanning Tool - IEEE802.11 WLAN analyzer |

1.5 References

| Reference | Source |
| --- | --- |
| IEEE/ANSI 830-1998 Standard for SRS | IEEE Standards Association |
| Kali Linux Tools Documentation | https://www.kali.org/tools/horst/ |
| Cameroon QoS Monitoring Platform Framework | ART/PATNUC 2026 |
| ITU-T SG12 QoS Guidelines | International Telecommunication Union |

2. General Description

2.1 Product Perspective

The Telecommunications Network Analysis System is a software-based solution that operates on the Kali Linux operating system. It utilizes existing command-line tools (horst, tcpdump, Wireshark) and Python-based analysis scripts to evaluate wireless network performance. The system functions within the following context:

2.2 Product Functions

The system shall perform the following high-level functions:

- Network Scanning and Discovery

  - Identify all wireless networks in the target area

  - Capture BSSID, ESSID, channel, and signal strength data

- Traffic Analysis

  - Capture network packets during peak and off-peak hours

  - Analyze protocol distribution and bandwidth usage

- Performance Assessment

  - Measure channel utilization and interference levels

  - Calculate key QoS metrics (latency, throughput, signal quality)

- Optimization Recommendations

  - Generate channel selection recommendations

  - Identify optimal access point placement

  - Suggest configuration improvements

- Report Generation

  - Create structured HTML/PDF reports of findings

  - Provide actionable recommendations for community stakeholders

2.3 User Characteristics

| User Type | Technical Level | Primary Tasks |
| --- | --- | --- |
| Student Researcher | Advanced | Execute scanning tools, analyze data, generate reports |
| Community Network Admin | Intermediate | Implement optimization recommendations |
| Local Resident | Basic | Understand report findings, follow basic configuration guidance |

2.4 Operating Environment

Hardware Requirements:

- Computer with Kali Linux installed (bare metal or VM with USB passthrough)

- Wireless network adapter supporting monitor mode

- Minimum 4GB RAM, 20GB storage

- Optional: RTL-SDR dongle for spectrum analysis

Software Requirements:

- Kali Linux (latest version)

- horst wireless analyzer

- tcpdump packet capture

- Wireshark for deep packet inspection

- Python 3.x with pandas, matplotlib, numpy

- Chaosreader for HTML report generation

2.5 Assumptions and Dependencies

Assumptions:

- The target wireless network belongs to the researcher or permission has been obtained for analysis

- The wireless adapter supports monitor mode and packet injection

- Network traffic during capture periods represents typical usage patterns

- The local community has basic WiFi infrastructure that can be optimized

Dependencies:

- Kali Linux tools must be properly installed and configured

- Regulatory compliance with Cameroon telecommunications laws

- Cooperation from local community members for extended testing periods

2.6 Constraints

| Constraint | Description |
| --- | --- |
| Legal | Analysis limited to authorized networks only |
| Technical | Monitor mode requires compatible wireless hardware |
| Temporal | Data collection limited to project timeline (4-6 weeks) |
| Geographic | Analysis focused on specific local community (1-3 locations) |
| Scope | 2.4 GHz and 5 GHz WiFi bands only (not cellular tower analysis) |

3. Specific Requirements

3.1 Functional Requirements

FR-1: Network Discovery and Scanning

| Element | Specification |
| --- | --- |
| ID | FR-1 |
| Description | The system SHALL discover all wireless networks within range of the scanning adapter |
| Priority | High |
| Input | Wireless interface in monitor mode |
| Processing | Execute horst with appropriate parameters to scan all channels |
| Output | List of discovered networks with BSSID, ESSID, channel, RSSI |
| Validation | System MUST detect at least 5 distinct networks in urban areas |

Implementation Command:

bash

sudo horst -i wlan0mon -V spectrum -C 1-11 -d 500

FR-2: Signal Strength Measurement

| Element | Specification |
| --- | --- |
| ID | FR-2 |
| Description | The system SHALL measure and record RSSI values for each detected station |
| Priority | High |
| Input | Active wireless network traffic |
| Processing | horst SHALL display signal (RSSI) values per station |
| Output | RSSI values in dBm for each connected station |
| Validation | RSSI measurements MUST be reproducible within ±3 dBm |

FR-3: Channel Utilization Analysis

| Element | Specification |
| --- | --- |
| ID | FR-3 |
| Description | The system SHALL calculate channel utilization by measuring packet airtime occupancy |
| Priority | High |
| Input | Channel traffic over specified time period |
| Processing | horst SHALL calculate usage by adding packet transmission time |
| Output | Channel utilization percentage per channel |
| Validation | Utilization MUST sum to ≤100% across all channels |

FR-4: Packet Capture and Storage

| Element | Specification |
| --- | --- |
| ID | FR-4 |
| Description | The system SHALL capture and store network packets for offline analysis |
| Priority | Medium |
| Input | Live network traffic |
| Processing | tcpdump SHALL capture packets to PCAP format |
| Output | PCAP files with timestamps and complete packet headers |
| Validation | Capture MUST include all packets without loss for given interface |

Implementation Command:

bash

sudo tcpdump -i wlan0mon -G 3600 -W 24 -C 100 -w capture_%Y%m%d_%H%M%S.pcap

FR-5: Traffic Protocol Analysis

| Element | Specification |
| --- | --- |
| ID | FR-5 |
| Description | The system SHALL analyze captured traffic to identify protocol distribution |
| Priority | Medium |
| Input | PCAP capture files |
| Processing | Python scripts using scapy or pyshark SHALL parse and categorize packets |
| Output | Protocol distribution percentages (HTTP, HTTPS, DNS, etc.) |
| Validation | Protocol classification MUST match Wireshark analysis |

FR-6: Report Generation

| Element | Specification |
| --- | --- |
| ID | FR-6 |
| Description | The system SHALL generate comprehensive reports of network analysis findings |
| Priority | High |
| Input | All collected data (scans, captures, analysis results) |
| Processing | Chaosreader SHALL create HTML reports from PCAP files |
| Output | Structured HTML report with graphs, tables, and recommendations |
| Validation | Report MUST include all required sections as specified in Section 4 |

Implementation:

bash

chaosreader capture_file.pcap

# Generates session_00.html, session_01.html, etc.

FR-7: Channel Optimization Recommendation

| Element | Specification |
| --- | --- |
| ID | FR-7 |
| Description | The system SHALL recommend optimal channel selection based on utilization data |
| Priority | High |
| Input | Channel utilization percentages and interference levels |
| Processing | Python algorithm SHALL identify least-congested channel |
| Output | Recommended channel number (1, 6, or 11 for 2.4 GHz) |
| Validation | Recommendation MUST improve throughput in validation testing |

FR-8: Historical Data Comparison

| Element | Specification |
| --- | --- |
| ID | FR-8 |
| Description | The system SHALL enable comparison of network performance across different time periods |
| Priority | Medium |
| Input | Multiple PCAP files from different times/days |
| Processing | Python analysis SHALL aggregate and compare metrics |
| Output | Time-series graphs showing performance trends |
| Validation | Comparison MUST identify peak vs. off-peak differences |

3.2 Non-Functional Requirements

NFR-1: Performance

| Element | Specification |
| --- | --- |
| ID | NFR-1 |
| Description | The system SHALL perform real-time analysis without significant packet loss |
| Target | < 5% packet loss during capture on supported hardware |
| Measurement | Compare captured vs. expected packet count |

NFR-2: Usability

| Element | Specification |
| --- | --- |
| ID | NFR-2 |
| Description | The system SHALL provide clear, actionable output for non-technical stakeholders |
| Target | Reports MUST include plain-language recommendations |
| Measurement | Local community members SHALL understand basic findings |

NFR-3: Portability

| Element | Specification |
| --- | --- |
| ID | NFR-3 |
| Description | The system SHALL run on standard Kali Linux installations |
| Target | No proprietary dependencies beyond Kali repositories |
| Measurement | Clean install on new Kali system MUST work within 30 minutes |

NFR-4: Documentation

| Element | Specification |
| --- | --- |
| ID | NFR-4 |
| Description | All analysis steps SHALL be documented for reproducibility |
| Target | Complete command logs and methodology section in final report |
| Measurement | Another researcher SHALL replicate results using documentation |

NFR-5: Regulatory Compliance

| Element | Specification |
| --- | --- |
| ID | NFR-5 |
| Description | The system SHALL operate within Cameroon telecommunications regulations |
| Target | No unauthorized network access or decryption attempts |
| Measurement | Analysis limited to open networks or authorized test networks |

3.3 Interface Requirements

I/F-1: Command Line Interface

| Element | Specification |
| --- | --- |
| ID | I/F-1 |
| Description | The system SHALL provide a command-line interface for tool execution |
| Commands | horst, tcpdump, chaosreader, Python scripts |
| Parameters | As defined in tool documentation |

I/F-2: File System Interface

| Element | Specification |
| --- | --- |
| ID | I/F-2 |
| Description | The system SHALL organize captured data in structured directories |
| Path | ~/telecom_analysis/captures/, ~/telecom_analysis/reports/ |
| Format | PCAP for captures, HTML for reports, CSV for metrics |

I/F-3: Python API

| Element | Specification |
| --- | --- |
| ID | I/F-3 |
| Description | The analysis system SHALL expose Python functions for metric calculation |
| Functions | calculate_channel_utilization(), analyze_signal_quality(), generate_optimization_report() |
| Input/Output | Pandas DataFrames for analysis, dictionaries for results |

3.4 Data Requirements

DR-1: Capture Data Schema

| Field | Type | Description |
| --- | --- | --- |
| timestamp | datetime | Time of capture |
| bssid | MAC address | Access point identifier |
| essid | string | Network name |
| channel | integer (1-11, 36-165) | Operating channel |
| rssi | integer (-100 to 0) | Signal strength in dBm |
| channel_util | float (0-100) | Percentage of channel busy time |

DR-2: Analysis Results Schema

| Field | Type | Description |
| --- | --- | --- |
| location_id | string | Test location identifier |
| time_period | string | "peak", "off-peak", or specific hours |
| avg_rssi | float | Mean signal strength |
| max_rssi | float | Best signal strength |
| min_rssi | float | Worst signal strength |
| recommended_channel | integer | Optimized channel selection |

3.5 Quality Attributes

| Attribute | Requirement | Measurement |
| --- | --- | --- |
| Accuracy | Signal measurements within ±3 dBm | Calibration against known source |
| Completeness | All required metrics captured | Checklist validation |
| Reproducibility | Same location yields similar results | Correlation coefficient > 0.9 |
| Timeliness | Real-time feedback during scanning | < 500ms display update |

4. Methodology for Analysis

4.1 Data Collection Protocol

Phase 1: Baseline Scan (Day 1-2)

bash

# Full spectrum scan during off-peak hours (6 AM - 8 AM)

sudo horst -i wlan0mon -V spectrum -o baseline_offpeak.txt

# Full spectrum scan during peak hours (6 PM - 9 PM)

sudo horst -i wlan0mon -V spectrum -o baseline_peak.txt

Phase 2: Extended Capture (Days 3-7)

bash

# 24-hour continuous capture with rotation

sudo tcpdump -i wlan0mon -G 3600 -W 24 -C 100 -w 24hr_capture.pcap

Phase 3: Targeted Analysis (Days 8-10)

- Identify problematic channels

- Measure specific access point performance

- Document interference sources

4.2 Optimization Validation Protocol

- Apply recommended changes to target network

- Repeat measurement using same methodology

- Compare before/after metrics

- Calculate improvement percentages

5. Deliverables

5.1 Code and Scripts

| File | Description |
| --- | --- |
| scan_automation.sh | Automated scanning script for horst and tcpdump |
| analyze_results.py | Python analysis script for metric calculation |
| generate_report.py | Report generation from collected data |
| channel_optimizer.py | Channel recommendation algorithm |

5.2 Documentation

| Document | Description |
| --- | --- |
| Installation Guide | Setup instructions for Kali Linux and tools |
| User Manual | How to run scans and interpret results |
| Technical Report | Complete analysis findings and recommendations |
| Presentation Slides | Academic defense presentation |

5.3 Data Artifacts

| Artifact | Format |
| --- | --- |
| Raw packet captures | PCAP files |
| Analysis outputs | CSV, JSON |
| Reports | HTML, PDF |
| Visualizations | PNG, SVG |

6. Success Criteria

The project SHALL be considered successful upon:

- Technical Completion

  - All functional requirements implemented and tested

  - Successful capture of network data for minimum 3 locations

  - Generation of complete analysis report

- Academic Requirements

  - Clear documentation of methodology

  - Statistical analysis of findings

  - Reproducible results

- Practical Impact

  - Actionable recommendations for local community

  - Demonstrated improvement (if implemented)

  - Contribution to understanding of Cameroon telecom challenges

7. Risks and Mitigation

| Risk | Probability | Impact | Mitigation |
| --- | --- | --- | --- |
| Wireless adapter lacks monitor mode | Medium | High | Verify compatibility before purchase; test with iw list |
| Low network activity during capture | Medium | Medium | Schedule captures during known peak hours; extend duration |
| Legal concerns about network analysis | Low | High | Only analyze authorized networks; document permissions |
| Hardware failure | Low | Medium | Have backup adapter available |
| Inconclusive optimization results | Medium | Low | Document findings even if no improvement; provide alternative recommendations |

8. Approval

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Student | [Your Name] |  |  |
| Academic Advisor | [Advisor Name] |  |  |
| Department Chair | [Chair Name] |  |  |

Appendix A: Sample Data Collection Checklist

- Kali Linux installed and updated

- Wireless adapter confirmed working with iwconfig

- Monitor mode enabled with airmon-ng start wlan0

- horst installed: sudo apt install horst

- tcpdump installed (default on Kali)

- Chaosreader installed

- Target location identified and permissions obtained

- Peak and off-peak time windows defined

- Data storage directory created

- Backup power source available for extended captures

Appendix B: Sample Python Analysis Functions

python

import pandas as pd

import numpy as np

from datetime import datetime

def parse_horst_output(filepath):

"""

Parse horst text output into structured DataFrame

Args:

filepath: Path to horst output file

Returns:

DataFrame with columns: bssid, essid, channel, rssi

"""

# Implementation here

pass

def calculate_channel_utilization(channel_data):

"""

Calculate channel utilization percentage

Args:

channel_data: DataFrame with channel utilization readings

Returns:

Dictionary with channel utilization by channel

"""

# Implementation here

pass

def recommend_channel(utilization_dict):

"""

Recommend optimal channel based on utilization

Args:

utilization_dict: {channel: utilization_percentage}

Returns:

Recommended channel number

"""

# For 2.4 GHz, only consider channels 1, 6, 11

relevant_channels = {1: utilization_dict.get(1, 0),

6: utilization_dict.get(6, 0),

11: utilization_dict.get(11, 0)}

return min(relevant_channels, key=relevant_channels.get)

def generate_optimization_report(dataframes, output_path):

"""

Generate comprehensive HTML report

Args:

dataframes: Dictionary of DataFrames with analysis results

output_path: Path for output HTML file

"""

# Implementation here

pass

Document prepared by: [Student Name]
Course: Application Development with Python
Date: April 2026