REPORT ON WATER QUALITY INDICATORS FOR COMMUNITY-BASED MONITORING PLATFORM

ACADEMIC DEFENSE PROJECT – BAMENDA, CAMEROON

Prepared for: Academic Supervisor
Subject: Drinking Water Health Indicators and Proposed Monitoring Framework

# 1. Executive Summary

This report presents research findings on drinking water health indicators suitable for a community-based software monitoring platform in Bamenda, Cameroon. Following the supervisor's directive, only indicators that directly determine whether water is healthy to drink are included. Ecological indicators (Dissolved Oxygen, Electrical Conductivity, Temperature) and aesthetic-only indicators (colour, Odor) have been excluded. The selected indicators are scientifically valid (WHO 2024 guidelines), locally appropriate, and practically collectible by community members, water workers, and health officials.

# 2. Research Methodology

Online Sources:

- WHO Guidelines for Drinking-water Quality: Small Water Supplies (2024)

- Peer-reviewed studies on water quality in Bamenda, Cameroon

- Water Quality Index research for African contexts

Field Consultations:

- Water maintenance officer at Akom drinking water dam

- Community members in Bamenda regarding water security issues

# 3. Proposed Drinking Water Health Indicators

## 3.1 Physical Indicators (Community-Level Reporting)

| Indicator | Reporting Method | Health Significance |
| --- | --- | --- |
| Turbidity | Visual: "Clear," "Cloudy," "Very muddy" | High turbidity harbours pathogens and protects them from disinfection |
| Visible Focal Contamination | Direct observation: sewage, dead animals near source | Immediate indicator of cholera, typhoid, and diarrheal disease risk |

## 3.2 Chemical Indicators (Water Worker-Level Testing)

| Indicator | Health-Based Target | Testing Method | Health Significance |
| --- | --- | --- | --- |
| pH | 6.5–8.5 | Test strips or meter | Extreme pH causes tissue damage to mouth, oesophagus, and stomach |
| Nitrate (NO₃⁻) | < 50 mg/L | Test strips | Causes methemoglobinemia ("blue baby syndrome") – fatal in infants |
| Nitrite (NO₂⁻) | < 3 mg/L | Test strips | More toxic than nitrate; causes blood oxygen transport failure |
| Free Chlorine Residual | 0.2–0.5 mg/L | DPD test kit | Ensures water remains pathogen-free from treatment to tap |

## 3.3 Biological Indicators (Health Official Reporting)

| Indicator | Detection Target | Health Implication |
| --- | --- | --- |
| E. coli / Fecal Coliforms | Fecal contamination | Gold standard – any detectable E. coli makes water unsafe to drink |
| Total Coliforms | General bacterial contamination | Indicates treatment or pipe integrity failure |
| Reported Diarrhea Cases | Waterborne disease incidence | Direct health outcome; elevation indicates community-wide drinking water failure |
| Cholera Cases | Vibrio cholerae | Directly linked to unsafe drinking water |
| Typhoid Cases | Salmonella typhi | Endemic in areas with unsafe drinking water |

## 3.4 Composite Indicator: Pass/Fail Framework

Safe to Drink: E. coli = 0 + Turbidity < 5 NTU + pH 6.5–8.5 + Nitrate < 50 mg/L
Unsafe to Drink: Any single parameter fails

# 4. Locations for Data Collection

| Location | Source Type | Priority | Rationale |
| --- | --- | --- | --- |
| Akom Drinking Water Dam | Surface water | HIGH | Initial research site; serves significant population |
| Pipe-borne water points | Municipal supply | HIGH | Primary source for 65% of households |
| Boreholes (3 Sub-Divisions) | Groundwater | MEDIUM | Secondary source for 18.6% of population |
| Springs and streams | Natural sources | MEDIUM | Used by 8.1%; often unprotected |
| Household wells | Groundwater | LOW | Very high contamination risk from nearby latrines |

Geographic Coverage: Bamenda I, Bamenda II, Bamenda III

# 5. Stakeholder Roles in Data Collection

| Stakeholder | Health Indicators to Provide | Frequency |
| --- | --- | --- |
| Water maintenance officers | pH, nitrate, nitrite, free chlorine, turbidity | Daily/Weekly |
| Community members | Turbidity (visual), visible faecal contamination | Real-time |
| Health officials | Diarrheal, cholera, typhoid cases; E. coli results | Weekly/Monthly |
| NGOs | Intervention records, chlorination status | As needed |
| Ministry of Water Resources | Regulatory compliance data | Monthly |

## Health Data-to-Action Pipeline:

Community Report (cloudy water + dead animal) → Dashboard Alert → Rapid E. coli test → Positive result → "UNSAFE TO DRINK" alert → Boil water advisory → Chlorination/repair → Follow-up confirmation

# 6. How This Platform Addresses Gaps

## 6.1 No Existing Platform in Cameroon

Research confirms no centralized drinking water health data platform exists in Cameroon. This would be a first-of-its-kind initiative.

## 6.2 Moving Beyond Hardware Limitations

| Hardware-Only Approach | Your Software Platform |
| --- | --- |
| Measures only chemical parameters | Integrates E. coli + turbidity + nitrate + health outcomes |
| No coordination mechanism | Direct alerts when water becomes unsafe |
| Expensive to scale | Scales via existing mobile phones |
| Requires spare parts supply chain | Remotely updatable |

# 7. Technical Recommendations

Data Validation Rules:

- E. coli: Any positive result → "UNSAFE TO DRINK"

- Turbidity: > 5 NTU → boiling advisory

- pH: < 6.0 or > 9.0 → health alert

- Nitrate: > 50 mg/L → warning for pregnant women and infants

- Free chlorine: < 0.2 mg/L → recontamination alert

Minimal Health Testing Kit for Water Workers:

- pH strips (range 4–10)

- Nitrate/nitrite combo test strips

- Free chlorine DPD test kit

- Turbidity tube

- E. coli rapid presence/absence test

# 8. Conclusion

Following the supervisor's directive to select only indicators concerning water quality healthy to drink, this report includes:

- Community-collectible: Turbidity (visual), visible fecal contamination

- Worker-collectible: pH, nitrate, nitrite, free chlorine, turbidity

- Health official-collectible: E. coli, total coliforms, diarrheal, cholera, typhoid cases

All selected indicators meet three criteria:

- Scientifically valid – WHO 2024 guidelines

- Directly health-relevant – Failure makes water unsafe to drink

- Actionable – Triggers specific public health response

The research confirms that a software-based monitoring platform focusing on drinking water health indicators is both feasible and urgently needed in Bamenda, Cameroon.

# 9. Request for Approval

Based on the evidence presented, I respectfully request approval to proceed with the software-based monitoring platform approach.

Next steps upon approval:

- Finalize health indicator pass/fail criteria

- Design data collection forms for each stakeholder type

- Develop prototype dashboard showing "SAFE" / "UNSAFE" status

- Pilot testing at Akom drinking water dam

# References

- WHO. (2024). Guidelines for drinking-water quality: small water supplies. Geneva.

- African Water Association. (2025). State of Urban Household Drinking Water Security Situation in Bamenda North West Region, Cameroon.

- African Water Association. (2025). Effectiveness and health risk assessment of drinking water from different sources treated by local household water treatment methods in Bamenda, Cameroon.

- ACECoR. (2025). Adapted Water Quality Indices for Water Quality Monitoring in Africa.