REPORT ON WATER QUALITY INDICATORS FOR COMMUNITY-BASED MONITORING PLATFORM

ACADEMIC DEFENSE PROJECT – BAMENDA, CAMEROON

Prepared for: Academic Supervisor
Subject: Research Findings on Water Quality Indicators and Proposed Monitoring Framework

# 1. Executive Summary

This report presents research findings from online literature review and field consultations regarding water quality indicators suitable for a community-based software monitoring platform. The investigation focused on identifying measurable parameters that can be effectively collected by diverse actors—community members, water workers, and health officials—to provide actionable insights for water quality management in Bamenda, Cameroon.

## 2. Research Methodology

## 2.1 Online Sources Consulted

- World Health Organization (WHO) Guidelines for Drinking-water Quality: Small Water Supplies (2024)

- Peer-reviewed studies on water quality in Bamenda, Cameroon

- Water Quality Index development research for African contexts

- Regional case studies from Nigeria and Ghana

- International water quality testing parameters

## 2.2 Field Consultations

- Water maintenance officer at Akom drinking water dam

- Community members in Bamenda regarding water security issues

## 3. Proposed Water Quality Indicators

Based on international standards and local feasibility, indicators are organized by who can collect them and their public health significance.

## 3.1 Physical Indicators (Community-Level Reporting)

These indicators can be reported by community members without specialized equipment:

| Indicator | Description | Reporting Method | Health Significance |
| --- | --- | --- | --- |
| Turbidity | Water clarity/clarity | Visual: "Clear," "Cloudy," "Very muddy" | High turbidity can harbor pathogens and reduce disinfection effectiveness |
| Colour | Unusual water coloration | Visual observation | Indicates organic matter, algae, or industrial contamination |
| Odor | Unusual smell | Sensory: "Normal," "Sewage smell," "Chemical smell" | Often indicates bacterial activity or chemical pollution |
| Visible Contamination | Floating debris, oil sheen, dead animals | Direct observation | Immediate health risk indicator |

## 3.2 Chemical Indicators (Water Worker-Level Testing)

These require basic field testing kits that water workers already possess or can be trained to use:

| Indicator | Target/Range | Testing Method | Why It Matters |
| --- | --- | --- | --- |
| pH | 6.5–8.5 | Test strips or meter | Affects disinfection; extreme pH causes health issues and pipe corrosion |
| Electrical Conductivity (EC) | < 2,500 µS/cm | Conductivity meter | Measures dissolved solids; high levels indicate pollution |
| Dissolved Oxygen (DO) | > 5 mg/L | DO meter or kit | Low DO indicates organic pollution and poor water quality |
| Temperature | Ambient (recorded) | Thermometer | Affects biological activity and disinfection |

## 3.3 Biological Indicators (Health Official Reporting)

These are collected by health officials tracking disease outbreaks:

| Indicator | What It Detects | Reporting Mechanism |
| --- | --- | --- |
| E. coli / Fecal Coliforms | Fecal contamination | Laboratory results or rapid test kits |
| Reported Diarrhea Cases | Waterborne disease incidence | Health facility records |
| Cholera/Typhoid Outbreaks | Serious waterborne diseases | Official outbreak reports |
| Staphylococcus, Salmonella, Shigella | Pathogen presence | Clinical laboratory data |

## 3.4 Composite Indicator: Water Quality Index (WQI)

Research from African contexts indicates that Water Quality Indices (WQIs) are valuable tools for combining multiple parameters into single, understandable values. The two most commonly adapted WQIs in Africa are:

- Weighted Arithmetic WQI (WAWQI) – Suitable for drinking water assessment

- Canadian Council of Ministers of Environment WQI (CCMEWQI) – Suitable for surface water monitoring

5. Locations for Data Collection

Based on existing research and local knowledge, the following locations should be prioritized for monitoring:

## 4.1 Primary Water Sources in Bamenda

| Location/Zone | Source Type | Priority Level | Rationale |
| --- | --- | --- | --- |
| Akom Drinking Water Dam | Surface water/Dam | HIGH | Your initial research site; serves significant population |
| Pipe-borne water distribution points | Municipal supply | HIGH | Primary water source for 65% of households |
| Boreholes across three Sub-Divisions | Groundwater | MEDIUM | Secondary source for 18.6% of population |
| Springs and streams | Natural sources | MEDIUM | Used by 8.1% of population; often unprotected |
| Household wells | Groundwater | LOW | Used by 8.3% of population; high contamination risk |

## 4.2 Geographic Coverage

The three Sub-Divisions of Bamenda to be covered:

- Bamenda I

- Bamenda II

- Bamenda III

# 5. Stakeholder Roles in Data Collection

Based on the WHO framework for small water supplies, multiple stakeholders contribute to water safety:

| Stakeholder | Role | Data to Provide | Frequency |
| --- | --- | --- | --- |
| Water maintenance officers | Professional testing | pH, EC, DO, temperature, turbidity | Daily/Weekly |
| Community members | Local observation | Visual quality, odor, color, pipe leaks | Real-time |
| Health officials | Disease surveillance | Diarrhea cases, outbreak reports, lab results | Weekly/Monthly |
| NGOs | Response coordination | Intervention records, repair status | As needed |
| Ministry of Water Resources | Policy oversight | Regulatory data, infrastructure updates | Monthly |

# 6. How This Platform Addresses Gaps

## 6.1 No Existing Platform in Cameroon

Research confirms that no centralized water quality data platform currently exists in Cameroon. Your proposed system would be a first-of-its-kind initiative.

## 6.2 Moving Beyond Hardware Limitations

| Hardware-Only Approach | Your Software Platform |
| --- | --- |
| Measures only chemical parameters | Integrates human observation + sensor data |
| No coordination mechanism | Direct NGO/technician alerts |
| Expensive to scale | Scales via existing mobile phones |
| Requires spare parts supply chain | Remotely updatable |
| Single point of failure | Distributed reporting network |

## 6.3 Data-to-Action Pipeline

This  platform enables:

Community Report → Dashboard Alert → NGO Notification → Repair Dispatch → Follow-up Confirmation

# 7. Technical Recommendations

## 7.1 Data Validation

Implement basic validation rules:

- pH must be between 0-14

- Temperature must be reasonable for tropical climate (15-40°C)

- Geographic coordinates required for each report

# 8. Conclusion

The research confirms that a software-based monitoring platform is both feasible and urgently needed in Bamenda. The WHO 2024 Guidelines explicitly support risk management approaches that combine multiple data sources—exactly what your platform provides.

The proposed indicators are:

- Scientifically valid (based on WHO and peer-reviewed research)

- Locally appropriate (informed by Bamenda-specific studies)

- Practically collectible (matched to stakeholder capabilities)

- Actionable (leading to specific interventions)

# 9. Request for Approval

Based on the evidence presented in this report, I respectfully request your approval to proceed with the software-based monitoring platform approach, with the indicator framework outlined above serving as the foundation for system design.

Next steps upon approval:

- Finalize indicator definitions and thresholds

- Design data collection forms for each stakeholder type

- Develop prototype dashboard

- Pilot testing at Akom drinking water dam

# References

- WHO. (2024). Guidelines for drinking-water quality: small water supplies. Geneva: World Health Organization.

- African Water Association. (2025). State of Urban Household Drinking Water Security Situation in Bamenda North West Region, Cameroon.

- African Water Association. (2025). Effectiveness and health risk assessment of drinking water from different sources treated by local household water treatment methods in Bamenda, Cameroon.

- ACECoR. (2025). Adapted Water Quality Indices: Limitations and Potential for Water Quality Monitoring in Africa.

- Missouri Department of Natural Resources. Water Quality Testing Parameters.